# Main Sim Paper Equations vs Current Code

I changed this one again so it now says what the constants actually mean and why those values were chosen, not just what the symbols are. I also corrected the values to match the current main.py, because some of them had drifted out of sync.

# Scope

This note compares the equations in /Users/rhyls/Desktop/fsc.pdf against the current main.py.

Scope is main.py only.

case1.py and case2.py are not part of this document.

# Important Note About The Pdf

The PDF text extraction damaged some Greek letters and some matrix formatting.

I rewrote the equations below in clean readable notation using the actual symbols.

For Eq. (35) and Eq. (38), the raw extraction of the closed-loop matrices is partially broken, so those two are described carefully instead of pretending the PDF extraction was perfect.

# Symbol Map

φ = roll = motion_state.roll_radians

θ = pitch = motion_state.pitch_radians

ψ = yaw = motion_state.yaw_radians

φ̇ = motion_state.roll_rate_rad_per_second

θ̇ = motion_state.pitch_rate_rad_per_second

ψ̇ = motion_state.yaw_rate_rad_per_second

φ̈ = roll angular acceleration

θ̈ = pitch angular acceleration

ψ̈ = yaw angular acceleration

u₁ = collective thrust

u₂ = pitch torque

u₃ = roll torque

u₄ = yaw torque

u₀₁ = vertical-channel control input from the paper

u₀₂ = pitch-channel linearized control input from the paper

u₀₃ = roll-channel linearized control input from the paper

u₀₄ = yaw-channel control input from the paper

# Variables Used In This Note

x, y, z = world position

ẋ, ẏ, ż = world velocity

ẍ, ÿ, z̈ = world acceleration

φ, θ, ψ = Euler angles

φ̇, θ̇, ψ̇ = Euler-angle rates

φ̈, θ̈, ψ̈ = Euler-angle accelerations

p, q, r = body rates used in the paper

T₁, T₂, T₃, T₄ = rotor thrusts

ωᵢ = rotor angular speed

m = mass

g = gravity

Iₓ, Iᵧ, I_z = principal inertias

L = arm length

k_T = thrust coefficient

k_r = yaw-moment ratio in the sim

γ_z, γ_ψ = paper guidance gains

P_z, D_z = paper altitude gains

P_ψ, D_ψ = paper yaw gains

# Actual Current Constants Used By The Sim

CONTROL_INPUT_LAG_SECONDS = 0.0

g = 9.81 m/s²

m = 1.6 kg

Iₓ = 0.045 kg·m²

Iᵧ = 0.052 kg·m²

I_z = 0.085 kg·m²

L = 0.28 m

k_T = 2.2 × 10⁻⁵

k_r = 0.018

τ_motor = 0.075 s

T_hover,per rotor = 3.924 N

ω_hover,per rotor ≈ 422.331 rad/s

a_xy,max = 8.4 m/s²

a_z,max = 5.2 m/s²

k_h = 4.2

k_v = 3.4

k_xy = 3.7

d_xy,base = 0.22 s⁻¹

d_xy,brake = 1.9 s⁻¹

v_xy,settle = 0.24 m/s

blend_manual = 0.0

blend_auto = 0.14

k_p,θ = 9.5

k_d,θ = 4.8

k_p,φ = 9.5

k_d,φ = 4.8

k_p,ψ = 6.0

k_d,ψ = 3.0

τ_θ,max = 0.42 N·m

τ_φ,max = 0.42 N·m

τ_ψ,max = 0.18 N·m

u₁,max = 37.6704 N

u₁,min = 5.4936 N

φ_max = 10°

θ_max = 7.5°

heading sign x = -1.0

heading sign y = 1.0

# Why These Values Were Chosen

Not all of these numbers came from the paper.

Some are physical modeling choices.

Some are control tuning choices.

Some are just gameplay and stability limits.

Physical-model choices

g = 9.81 m/s²

This is standard gravity.

m = 1.6 kg

Chosen to make the drone feel like a medium quadrotor, not a tiny toy drone and not a heavy industrial one.

Iₓ = 0.045, Iᵧ = 0.052, I_z = 0.085

Chosen so yaw is harder to spin than roll or pitch, which is normal for a quadrotor.

I_z is larger because spinning around the vertical axis usually feels more resistant.

L = 0.28 m

Chosen as a medium arm length so torque from rotor thrust is believable without making the drone huge.

k_T = 2.2 × 10⁻⁵

Chosen so the hover rotor speed comes out in a believable range instead of needing absurdly high or absurdly low rotor RPM.

k_r = 0.018

Chosen so yaw exists, but stays weaker than the main roll/pitch channels.

Derived values

T_hover,per rotor = 3.924 N

Not hand-picked directly.

This comes from mg / 4, so each rotor carries one quarter of the weight at hover.

ω_hover,per rotor ≈ 422.331 rad/s

Also not hand-picked directly.

This comes from the hover thrust and the thrust coefficient k_T.

u₁,max = 37.6704 N

Chosen as 2.4 × mg, which gives enough extra thrust above hover for climbing and recovery.

u₁,min = 5.4936 N

Chosen as 0.35 × mg, so the drone can reduce thrust a lot without fully killing the control authority.

Actuator feel

τ_motor = 0.075 s

Chosen to make the drone feel heavier and less twitchy.

Bigger τ_motor means more laggy prop response.

Smaller τ_motor would feel snappier but more arcade-like.

Motion and controller limits

a_xy,max = 8.4 m/s²

Chosen so horizontal response is strong enough to stop drift, but not so strong that the drone snaps unnaturally.

a_z,max = 5.2 m/s²

Chosen lower than horizontal so vertical motion stays controlled and does not bounce too hard.

φ_max = 10°

Chosen to keep roll visually believable and to prevent huge side-leaning.

θ_max = 7.5°

Chosen even smaller than roll so forward/back motion stays controlled and less aggressive.

τ_θ,max = 0.42 N·m

τ_φ,max = 0.42 N·m

τ_ψ,max = 0.18 N·m

Chosen so yaw remains weaker than pitch and roll, and pitch/roll remain responsive without becoming unstable.

Guidance and feedback gains

k_h = 4.2

This is the altitude guidance aggressiveness.

Chosen so altitude errors are corrected quickly without feeling too jumpy.

k_v = 3.4

This is the vertical rate correction gain.

Chosen to support hover stability after the bounded-guidance step.

k_xy = 3.7

Chosen so horizontal speed errors are corrected strongly enough to reduce drift.

k_p,θ = 9.5, k_d,θ = 4.8

k_p,φ = 9.5, k_d,φ = 4.8

Chosen as balanced PD gains so pitch and roll track target angles quickly without obvious oscillation.

k_p,ψ = 6.0, k_d,ψ = 3.0

Chosen lower than pitch/roll because yaw should feel steadier and less twitchy.

Gameplay damping and stop behavior

d_xy,base = 0.22 s⁻¹

Small always-on horizontal damping so the drone does not feel frictionless.

d_xy,brake = 1.9 s⁻¹

Extra damping when you release the stick, to reduce long sliding stops.

v_xy,settle = 0.24 m/s

If speed gets below this while braking, the sim snaps the residual drift to zero.

This is a gameplay choice, not a paper value.

blend_manual = 0.0

Chosen so wind does not secretly bias manual movement commands.

blend_auto = 0.14

Chosen so automation can feel a little more influenced by wind without losing control.

Sign conventions

heading sign x = -1.0

heading sign y = 1.0

These are not physics tunings.

They are coordinate-convention fixes so heading math lines up with the world axes and model orientation.

# What I Mean By Direct, Very Close, And Partial

Direct match means the code is using the same mathematical form as the paper, with only naming, ordering, or bookkeeping differences.

Very close means the code is using the same core equation, but there are a few implementation differences such as clamping, using post-lag thrust instead of ideal thrust, or integrating stored Euler rates directly.

Partial or simplified means the code is using the same physics idea, but not the same equation structure as the paper.

# Exact Paper Equations Vs Current Code

## Eq. (2): Translational Dynamics

Paper, rewritten in clean notation:

ẍ = (u₁ / m) [cos(ψ) sin(φ) + sin(ψ) sin(θ) cos(φ)]

ÿ = (u₁ / m) [sin(ψ) sin(φ) - cos(ψ) sin(θ) cos(φ)]

z̈ = (u₁ / m) cos(θ) cos(φ) - g

Code:

main.py:3548-3554

Comparison:

This is a direct match.

world_accel_x, world_accel_y, and world_accel_z use this same thrust-projection structure.

Differences:

the code uses the actual post-lag collective thrust collective_thrust_newtons / m, not the ideal pre-lag u₁ / m

extra gameplay damping is applied immediately after this acceleration is computed

## Eq. (8): Attitude Dynamics

Paper, rewritten in clean notation:

θ̈ = ((Iᵧ - I_z) / Iₓ) φ̇ ψ̇ + u₂ / Iₓ

φ̈ = ((I_z - Iₓ) / Iᵧ) θ̇ ψ̇ + u₃ / Iᵧ

ψ̈ = u₄ / I_z

Code:

main.py:3510-3520

Comparison:

This is very close.

pitch_accel, roll_accel, and yaw_accel are the implemented angular accelerations.

What is different:

the code uses actual torques reconstructed after rotor lag, not the ideal control torques before lag

the code stores Euler rates directly instead of separately storing p, q, r

the code clamps φ and θ after integration to ±18° and ±12°

## Eq. (9): Euler-Rate to Body-Rate Relationship

Paper, rewritten in clean notation:

[φ̇, θ̇, ψ̇]^T = S(φ, θ) [p, q, r]^T

Code:

No exact direct implementation.

Comparison:

The sim does not explicitly store separate body rates p, q, r.

Instead, it stores Euler-angle rates directly in DroneMotionState.

So this exact matrix relationship is not coded as written.

## Eq. (10): Small-Angle Approximation

Paper, rewritten in clean notation:

φ̇ ≈ p

θ̇ ≈ q

ψ̇ ≈ r

Code:

main.py:3522-3538

Comparison:

This is the approximation the current sim is effectively using.

What is different:

the paper presents this as an approximation step

the code effectively treats this approximation as the runtime model for the Euler-angle rates

## Eq. (11): Full Nonlinear State Model

Paper, rewritten in clean notation:

ẋ = f(x, u)

Code:

main.py:3443-3601

Comparison:

The sim does not implement Eq. (11) as one big state-space block.

Instead, it implements the same model in pieces:

altitude and thrust command

torque command

rotor lag

attitude acceleration

world-frame translational acceleration

velocity and position integration

## Eq. (12): Rotor Thrust Mixing

Paper, rewritten in clean notation:

[u₁, u₂, u₃, u₄]^T = M [T₁, T₂, T₃, T₄]^T

Tᵢ = k_T ωᵢ²

Code:

main.py:2304-2326

main.py:2329-2337

Comparison:

This is a structural match.

What is different:

the paper writes the mixing as a matrix equation

the code expands the same logic manually into front/back/left/right expressions

the exact rotor order and sign convention depend on the sim's axis labeling

the sim uses k_r = 0.018 for the yaw-moment ratio

## Eq. (14): DC Motor Transfer Function

Paper:

Ω(s) / Vₐ(s) = kₜ / ((L s + R)(J s + B) + kₑ kₜ)

Code:

main.py:2340-2361

Comparison:

This is a simplified match, not an exact one.

What is different:

the paper motor model depends on L, R, J, B, kₑ, kₜ

the code collapses all of that into a first-order lag with τ_motor = 0.03 s

## Eq. (16): Linearized Pitch-Channel Model

Paper, rewritten in clean notation:

ÿ ≈ -g θ

z̈ ≈ u₁ / m - g

θ̈ ≈ u₂ / Iₓ

Code:

main.py:3420-3424

main.py:3548-3554

Comparison:

This is a partial but close conceptual match.

What is different:

the paper is a linearized state-space hover model

the code uses the same small-angle relationship inside a nonlinear controller loop

the code clamps both desired and actual pitch angles

in the controller, the same idea appears as desired_pitch_radians = -desired_body_forward_accel / g

## Eq. (19): Linearized Roll-Channel Model

Paper, rewritten in clean notation:

ẍ ≈ g φ

z̈ ≈ u₁ / m - g

φ̈ ≈ u₃ / Iᵧ

Code:

main.py:3415-3419

main.py:3548-3554

Comparison:

This is a partial but close conceptual match.

What is different:

the paper is a linearized state-space hover model

the code uses the same small-angle relationship inside a nonlinear controller loop

the code clamps both desired and actual roll angles

in the controller, the same idea appears as desired_roll_radians = desired_body_right_accel / g

## Eq. (21): Yaw Dynamics

Paper:

ψ̈ = u₄ / I_z

Code:

main.py:3520

Comparison:

This is basically exact.

What is different:

the sim uses the actual post-lag yaw torque recovered from rotor thrusts

the code wraps ψ after integration so heading stays bounded

## Eq. (22): Full Linearized Quadrotor Model

Paper:

ẋ = A x + B u

Code:

No direct A, B matrix implementation.

Comparison:

The sim uses the same channel logic, but not in one combined matrix form.

## Eq. (32): Hovering-Mode Local Dynamics

Paper:

z̈ = u₀₁

Code:

main.py:3449-3453

main.py:3554

Comparison:

This is a partial match.

What is different:

the paper writes the vertical channel as a local double integrator

the code turns that vertical command into collective thrust

the actual final z̈ still depends on thrust projection through cos(θ) cos(φ)

## Eq. (33): Bounded Guidance Law

Paper, rewritten in clean notation:

ż_d = -γ_z [z̃ / (1 + |z̃|)]

h(x) = x / (1 + |x|)

Code:

main.py:2292-2293

main.py:3443-3448

Comparison:

This is one of the strongest direct matches in the sim.

What is different:

the paper writes it as a guidance law using γ_z

the code uses DRONE_HOVER_GUIDANCE_GAIN = 4.2

the code then clamps the resulting ż_d

## Eq. (34): Altitude PD Channel

Paper, rewritten in clean notation:

u₀₁ = P_z (ż_d - ż) + D_z (z̈_d - z̈)

Code:

main.py:3449-3453

Comparison:

This is simplified compared to the paper.

The code keeps the velocity-error part:

a_z,des = k_v (ż_d - ż)

What is different:

the paper includes an extra acceleration-error term

the code does not explicitly implement that extra term

the code clamps the resulting desired vertical acceleration to ±5.2 m/s²

## Eq. (35): Closed-Loop Altitude System

Paper:

closed-loop matrix form derived from Eq. (32) to Eq. (34)

Code:

No direct matrix form is coded.

Comparison:

The raw PDF extraction damaged the exact matrix layout.

The sim implements controller behavior that creates a related closed loop, but it does not explicitly code Eq. (35) as a matrix system.

## Eq. (36): Yaw Guidance Law

Paper, rewritten in clean notation:

ψ̇_d = -γ_ψ (ψ_T - ψ_A)

Code:

main.py:3426-3441

Comparison:

This is not an exact match.

What is different:

the paper commands a desired yaw rate ψ̇_d

in manual mode, the code mostly commands a desired yaw angle from camera_angle

in automation mode, yaw follows command direction or velocity direction instead

## Eq. (37): Yaw PD Channel

Paper, rewritten in clean notation:

u₀₄ = P_ψ (ψ̇_d - ψ̇_A) + D_ψ d/dt (ψ̇_d - ψ̇_A)

Code:

main.py:3485-3494

Comparison:

This is not an exact match.

The code uses PD on yaw angle error and yaw rate:

u₄ = I_z [k_p,ψ (ψ_des - ψ) - k_d,ψ ψ̇]

What is different:

the paper is PD on yaw-rate tracking

the code is PD on yaw-angle error plus yaw-rate damping

## Eq. (38): Closed-Loop Yaw System

Paper:

closed-loop yaw matrix derived from Eq. (36) and Eq. (37)

Code:

No direct matrix form is coded.

Comparison:

The PDF extraction damaged this matrix too.

The sim also uses a different yaw controller structure, so Eq. (38) is not directly implemented.

## Eq. (40): Position-Attitude Kinematic Relationship

Paper, rewritten in clean notation:

[ẍ, ÿ]^T = g R(ψ) [θ, φ]^T

Code:

main.py:3415-3424

main.py:3548-3553

Comparison:

This is conceptually very close.

What is different:

the paper writes the relation as a compact kinematic equation

the code uses the same idea through the desired-angle controller plus the full thrust projection

in the sim, horizontal acceleration is also affected by rotor lag and extra damping

# Direct, Partial, And Not Exact

Strongest direct matches:

Eq. (2)

Eq. (12)

Eq. (21)

the bounded-guidance structure in Eq. (33)

Very close but not exact:

Eq. (8)

Partial or simplified matches:

Eq. (14)

Eq. (16)

Eq. (19)

Eq. (32)

Eq. (34)

Eq. (40)

Not implemented exactly as written:

Eq. (9)

Eq. (22)

Eq. (35)

Eq. (36)

Eq. (37)

Eq. (38)

# Biggest Scientific Differences

The sim uses the small-angle assumption from Eq. (10) instead of the full Eq. (9) transformation.

The rotor lag is simplified to first-order response instead of the full DC motor transfer function in Eq. (14).

The yaw controller is not the same controller as Eq. (36) to Eq. (38).

The altitude channel is close to the paper, but still simplified compared to the full Eq. (34) form.

The sim also keeps extra braking damping and a stop-settle rule that are not from the paper.

# Bottom Line

The main sim is clearly paper-inspired.

The translational thrust projection, attitude dynamics, rotor mixing, yaw acceleration, and bounded guidance are the strongest paper-to-code links.

But it is not a line-by-line reproduction of the paper.

The biggest simplifications are:

direct Euler-rate integration

simplified motor lag

a different yaw controller

extra gameplay damping and stop-settle logic