Updated Main Sim Formula Comparison

Technical paper-to-code and implementation-equation audit for current main.py

Updated: June 8, 2026

Primary code: /Users/rhyls/Desktop/Summer26 FSC/human trust w auto sys/real sim/main.py

Helper code: /Users/rhyls/Desktop/Summer26 FSC/human trust w auto sys/real sim/simulation_helpers.py

Older notes used as starting point: MAIN_SIM_EQUATIONS_AND_CODE.md and MAIN_SIM_EULER_DYNAMICS_BEFORE_AFTER.md

Scope: Current main sim formulas: quadrotor dynamics, wind/fire spread, detection, suppression, mapping, automation, and performance score.

# Technical Bottom Line

The updated code is no longer just a visual banking model. The main drone and follower drones now use stored Euler attitude states, rotor thrust mixing, first-order motor lag, torque-driven attitude dynamics, and thrust projection into world-frame acceleration.

The paper-to-code match is strongest for translational thrust projection, attitude acceleration, rotor mixing, yaw acceleration, and bounded altitude guidance. It is intentionally simplified for the motor model, yaw guidance, full Euler/body-rate mapping, and full state-space matrix forms.

The new code also adds formula-heavy wildfire-system logic: wind-biased discrete hotspot spread, range/altitude-based probabilistic detection, suppression work accumulation, mapping coverage, multi-drone effectiveness, and a four-term performance score.

| Area | Current status | Main code references |
| --- | --- | --- |
| Quadrotor control/dynamics | Implemented twice: reusable follower pipeline and inline lead-drone pipeline use the same equation structure. | main.py:3303-3595; main.py:8663-8963 |
| Rotor mixing/motor lag | Current and correct. Motor lag uses τ_motor = 0.075 s, not 0.03 s. | main.py:3199-3268 |
| Wind/fire spread | Old spread note is stale; current interval is 1.9 s, one event per tick, 4.8-12.5 m base jumps. | main.py:447-472; main.py:8398-8535 |
| Detection/suppression | Detection is per-frame probability with range and altitude factors; suppression is accumulated work. | main.py:8216-8375 |
| Performance score | Four weighted terms: detection, coverage, suppression progress, and fire-out control. | main.py:4179-4350 |

# Current Constants Snapshot

Drone constants from current main.py:159-198.

| Symbol/code constant | Current value | Meaning |
| --- | --- | --- |
| g | 9.81 m/s² | Gravity. |
| m | 1.6 kg | Drone mass. |
| I_x, I_y, I_z | 0.045, 0.052, 0.085 kg·m² | Principal inertias used by attitude dynamics. |
| L_arm | 0.28 m | Rotor arm length used for roll/pitch torques. |
| k_T | 2.2e-5 | Rotor thrust coefficient in T_i = k_T ω_i². |
| k_r | 0.018 | Yaw moment ratio used for rotor yaw torque. |
| τ_motor | 0.075 s | First-order rotor speed lag time constant. |
| T_hover,rotor | 3.924 N | m g / 4. |
| ω_hover,rotor | ≈ 422.331 rad/s | sqrt(T_hover / k_T). |
| u₁,min / u₁,max | 5.4936 N / 37.6704 N | 0.35 m g and 2.4 m g collective thrust bounds. |

Fire/detection/suppression constants from current main.py:405-446.

| Symbol/code constant | Current value | Meaning |
| --- | --- | --- |
| R_det | 15.0 m | Hotspot detection radius. |
| λ_det | 1.8 s⁻¹ | Base detection probability rate. |
| h_min | 4.5 m | Minimum scan altitude above hotspot ground. |
| T_contain | 30.0 s | Suppression work needed to mark a detected active hotspot contained. |
| T_extinguish | 54.0 s | Additional suppression work after containment to mark it out. |
| T_burnout | 180.0 s | Undetected/uncontrolled active hotspot burnout threshold. |
| r_ground | 0.55 s/s | Ground firefighter suppression work rate after response delay. |
| r_water | 0.9 s/s per engaged water drone | Water drone suppression work boost. |
| p_reignite | 0.0035 s⁻¹ | Per-second reignition probability for suppressed-out hotspots. |

Wind/fire spread constants from current main.py:447-472.

| Symbol/code constant | Current value | Meaning |
| --- | --- | --- |
| Δt_spread | 1.9 s | Spread check interval. |
| N_spread | 1 event/tick | Maximum attempted new hotspots per spread tick. |
| d_min, d_max | 4.8 m, 12.5 m | Base random spread jump distance range. |
| s_cell | 10.0 m | Burn-grid cell size. |
| v_w,base | 4.0 m/s | Base wind speed. |
| gust speed range | ±1.8 m/s | Current wind speed is max(0, base + gust). |
| b_wind | 0.08 per (m/s) | Directional-bias probability factor for fire spread. |
| χ_cone | ±55° | Wind-biased spread cone half-angle. |
| G_down / G_up | 0.68 / 1.0 | Downwind distance gain and upwind distance penalty. |

# Symbol Glossary

| Symbol | Meaning | Code equivalent |
| --- | --- | --- |
| x, y, z | World position in meters. | drone.getX(), getY(), getZ() |
| v_x, v_y, v_z | World velocity components. | motion_state.velocity.x/y/z |
| v_des | Desired world-frame velocity before wind blending. | desired_world_vx/y/vz |
| v_cmd | Commanded velocity after optional wind blending. | commanded_vx/y |
| b_w | Wind command blend coefficient. | wind_command_blend |
| e_R | Horizontal body-right unit axis. | body_right |
| e_F | Horizontal body-forward unit axis. | body_forward |
| v_R, v_F | Velocity resolved onto body-right and body-forward axes. | current_body_right/forward_speed |
| a_x, a_y, a_z | World acceleration components. | world_accel_x/y/z |
| a_R,d, a_F,d | Desired body-right/body-forward accelerations. | desired_body_right/forward_accel |
| a_xy,max | Maximum desired horizontal acceleration. | DRONE_MAX_HORIZONTAL_ACCEL_METERS_PER_SECOND_SQ |
| a_z,max | Maximum desired vertical acceleration. | DRONE_MAX_VERTICAL_ACCEL_METERS_PER_SECOND_SQ |
| φ | Roll angle in radians. | motion_state.roll_radians |
| θ | Pitch angle in radians. | motion_state.pitch_radians |
| ψ | Yaw/heading angle in radians. | motion_state.yaw_radians |
| φ_d, θ_d, ψ_d | Desired roll, pitch, and yaw commands. | desired_roll/pitch/target_yaw_radians |
| φ̇, θ̇, ψ̇ | Euler-angle rates. | roll_rate, pitch_rate, yaw_rate |
| φ̈, θ̈, ψ̈ | Euler-angle accelerations. | roll_accel, pitch_accel, yaw_accel |
| z_target | Altitude target used by the hover controller. | target_altitude_meters |
| e_z | Altitude error z - z_target. | altitude_error |
| ż_d | Desired vertical rate. | desired_vertical_rate |
| k_h, k_v | Altitude guidance and vertical-rate gains. | DRONE_HOVER_GUIDANCE_GAIN / RATE_GAIN |
| u₁ | Collective thrust. | collective_thrust_newtons |
| u₂ / τ_θ | Pitch torque. | pitch_torque_nm |
| u₃ / τ_φ | Roll torque. | roll_torque_nm |
| u₄ / τ_ψ | Yaw torque. | yaw_torque_nm |
| S_fb, S_lr | Intermediate front/back and left/right rotor-thrust sums. | front_back_sum / left_right_sum |
| T_f, T_b, T_l, T_r | Front, back, left, and right rotor thrusts. | front/back/left/right_thrust |
| ω_i | Rotor i angular speed. | rotor_omegas_rad_per_second[i] |
| k_T | Rotor thrust coefficient. | DRONE_ROTOR_THRUST_COEFFICIENT |
| k_r | Yaw-moment ratio. | DRONE_ROTOR_YAW_MOMENT_RATIO |
| τ_motor | Rotor first-order lag time constant. | DRONE_MOTOR_TIME_CONSTANT_SECONDS |
| α | Rotor low-pass blend 1-exp(-Δt/τ_motor). | blend in low_pass_value |
| h(e) | Bounded guidance function e/(1+¦e¦). | bounded_guidance(error_value) |
| sat(x,a,b) | Clamp x to interval [a,b]. | clamp(value, minimum, maximum) |
| Δt / dt | Simulation frame time step. | dt |
| χ_w | Wind direction heading. | wind_direction_degrees/radians |
| v_w | Wind speed. | wind_speed_mps |
| p_bias | Probability of drawing a spread direction from the downwind cone. | wind_direction_bias |
| α_spread | Sampled fire spread angle. | spread_angle |
| α_w | Wind alignment cos(spread_angle - wind_heading). | wind_alignment |
| d₀, d | Base and wind-adjusted fire spread distance. | spread_distance |
| R_det | Hotspot detection radius. | FIRE_HOTSPOT_DETECTION_RADIUS_METERS |
| d_h | Horizontal drone-to-hotspot distance. | horizontal_distance |
| h_rel | Drone altitude above hotspot ground. | altitude_above_hotspot |
| P_det | Per-frame detection probability. | detection_probability |
| f_map,i | Mapping completion fraction for hotspot i. | hotspot_mapping_fraction(hotspot) |
| W_i | Suppression work accumulated for hotspot i. | hotspot.suppression_work_seconds |
| r_supp | Current suppression-work rate. | current_suppression_rate_per_second |
| M1-M4 | Performance sub-scores. | m1_score ... m4_score |
| E_role(n) | Survey or water role effectiveness multiplier. | team_role_effectiveness(role_drone_count) |
| N_total | Total drone count selected for the run. | team_total_drone_count |

# Paper Equation Comparison

This section updates the earlier paper-equation comparison using the current line locations. The paper equations are the same equation set transcribed in the older note; the verification here is against the current source code.

| Paper equation | Current implementation | Match level | Technical note |
| --- | --- | --- | --- |
| Eq. (2) translational thrust projection | main.py:3487-3493 and 8874-8880 | Direct core match | The same nonlinear thrust projection is used, then extra damping/limits are applied. |
| Eq. (8) attitude dynamics | main.py:3449-3459 and 8836-8846 | Very close | Uses post-lag reconstructed torques and stored Euler rates. |
| Eq. (9) Euler/body-rate map | No explicit p,q,r state | Not exact | The sim uses the small-angle assumption and integrates Euler rates directly. |
| Eq. (10) small-angle approximation | main.py:3461-3477 and 8848-8864 | Runtime assumption | The approximation is effectively built into the state update. |
| Eq. (12) rotor mixing | main.py:3199-3232 | Structural match | Matrix is expanded manually in front/back/left/right rotor variables. |
| Eq. (14) DC motor model | main.py:3235-3260 | Simplified | Full motor electrical model is collapsed to first-order rotor speed lag with τ_motor = 0.075 s. |
| Eq. (16)/(19) hover translation channels | main.py:3353-3373 and 8741-8761 | Partial/close | Uses a ≈ g angle logic for desired roll/pitch, with clamps and nonlinear projection. |
| Eq. (21) yaw acceleration | main.py:3459 and 8846 | Direct core match | ψ̈ = τ_ψ / I_z after rotor lag. |
| Eq. (22), (35), (38) state-space matrices | No A/B matrix blocks | Not implemented as written | Equivalent behaviors are coded procedurally, not as closed-loop matrices. |
| Eq. (33) bounded altitude guidance | main.py:3187-3188; 3378-3388; 8769-8779 | Direct structure | h(e)=e/(1+¦e¦), then rate/acceleration clamps. |
| Eq. (36)/(37) yaw guidance/PD | main.py:3420-3429 and 8811-8820 | Different controller | Code uses yaw-angle error plus yaw-rate damping, not the paper's yaw-rate tracking form. |
| Eq. (40) position-attitude relation | main.py:3353-3373; 3487-3493; 8741-8761; 8874-8880 | Conceptually close | Desired attitude is generated from desired horizontal acceleration; actual acceleration comes from thrust projection. |

# Current Quadrotor Control And Dynamics

## Commanded Velocity And Body Axes

Manual control converts keyboard input into world-frame desired velocity using camera heading. Automation computes desired velocity from patrol, hotspot, orbit, and avoidance rules. Wind is optionally blended into the command.

v_cmd,x = v_des,x + b_w v_w,x
v_cmd,y = v_des,y + b_w v_w,y
e_R = [cos ψ, sin ψ]
e_F = [-sin ψ, cos ψ]
v_R = v · e_R,   v_F = v · e_F

Here b_w is the wind command blend: 0.0 in manual mode and 0.14 for automation. e_R is the drone's body-right axis and e_F is the body-forward axis in the horizontal plane.

## Desired Horizontal Acceleration To Desired Attitude

a_R,d = sat(k_xy (v_R,d - v_R), -a_xy,max, a_xy,max)
a_F,d = sat(k_xy (v_F,d - v_F), -a_xy,max, a_xy,max)
φ_d = sat(a_R,d / g, -φ_max, φ_max)
θ_d = sat(-a_F,d / g, -θ_max, θ_max)

This is the small-angle hover idea: horizontal acceleration is approximately gravity times attitude angle. Positive desired right acceleration creates positive roll; positive desired forward acceleration creates negative pitch in this coordinate convention.

## Altitude Guidance And Collective Thrust

e_z = z - z_target
h(e_z) = e_z / (1 + |e_z|)
ż_d = sat(-k_h h(e_z), -v_z,max, v_z,max)
a_z,d = sat(k_v (ż_d - ż), -a_z,max, a_z,max)
u₁,cmd = sat(m (g + a_z,d) / max(0.3, cos θ cos φ), u₁,min, u₁,max)

The lift projection denominator compensates for tilt so altitude control does not lose all vertical authority while the drone is pitched or rolled.

## Attitude PD Torques

τ_θ,cmd = sat(I_x [k_pθ(θ_d - θ) - k_dθ θ̇], -τ_θ,max, τ_θ,max)
τ_φ,cmd = sat(I_y [k_pφ(φ_d - φ) - k_dφ φ̇], -τ_φ,max, τ_φ,max)
e_ψ = wrap(ψ_d - ψ)
τ_ψ,cmd = sat(I_z [k_pψ e_ψ - k_dψ ψ̇], -τ_ψ,max, τ_ψ,max)

This is PD attitude tracking. Pitch and roll track desired angles; yaw tracks a desired heading and damps yaw rate.

## Rotor Mixing And Motor Lag

S_fb = (u₁ - u₄/k_r) / 2
S_lr = (u₁ + u₄/k_r) / 2
T_f = max(0, (S_fb + u₂/L_arm) / 2)
T_b = max(0, (S_fb - u₂/L_arm) / 2)
T_l = max(0, (S_lr + u₃/L_arm) / 2)
T_r = max(0, (S_lr - u₃/L_arm) / 2)

ω_i,d = sqrt(max(0, T_i,d) / k_T)
α = 1 - exp(-Δt / τ_motor)
ω_i ← ω_i + α(ω_i,d - ω_i)
T_i = k_T ω_i²

The inverse reconstruction used after lag is u₁ = T_f+T_b+T_l+T_r, u₂ = L_arm(T_f-T_b), u₃ = L_arm(T_l-T_r), and u₄ = k_r(-T_f-T_b+T_l+T_r).

## Attitude And Translational Dynamics

θ̈ = ((I_y - I_z)/I_x) φ̇ ψ̇ + τ_θ/I_x
φ̈ = ((I_z - I_x)/I_y) θ̇ ψ̇ + τ_φ/I_y
ψ̈ = τ_ψ/I_z

ẍ = (u₁/m)[cosψ sinφ + sinψ sinθ cosφ]
ÿ = (u₁/m)[sinψ sinφ - cosψ sinθ cosφ]
z̈ = (u₁/m) cosθ cosφ - g

d_xy = d_base + d_brake β_h
β_h = sat(1 - ||v_cmd,h|| / v_h,max, 0, 1)
d_z = d_z,active + d_z,passive β_z
β_z = sat(1 - |v_cmd,z| / v_z,max, 0, 1)
ẍ ← ẍ - d_xy v_x,   ÿ ← ÿ - d_xy v_y,   z̈ ← z̈ - d_z v_z

The paper dynamics are present, but damping, speed caps, angle clamps, boundary clamps, and stop-settle rules are sim/gameplay stabilizers rather than paper equations.

# Wind And Fire Spread

## Wind State

χ_w = (χ_base + χ_gust) mod 360°
v_w = max(0, v_base + v_gust)
v_w,x = sin(χ_w) v_w
v_w,y = cos(χ_w) v_w

The gust heading and speed move toward random targets using rate limits: 18°/s for direction and 1.5 m/s per second for speed.

## Discrete Hotspot Spread

Fire spread is not a continuous PDE flame-front model. It is a discrete hotspot-spawning model that runs every 1.9 sim seconds and can create at most one new hotspot per spread tick if a valid candidate is found.

p_bias = min(0.9, v_w · 0.08)
if U(0,1) < p_bias: α_spread ~ Uniform(χ_w - 55°, χ_w + 55°)
else: α_spread ~ Uniform(0, 2π)
d₀ ~ Uniform(4.8 m, 12.5 m)
a_w = cos(α_spread - χ_w)

if a_w > 0: d = d₀ + a_w v_w · 0.68
if a_w < 0: d = d₀ + a_w v_w · 1.0
d ← sat(d, 0.6 · 4.8, 1.8 · 12.5)
x_new = x_source + cos(α_spread)d
y_new = y_source + sin(α_spread)d

| Spread quantity | Current value |
| --- | --- |
| No-wind per-hop distance | 4.8-12.5 m |
| No-wind equivalent hop speed | 2.53-6.58 m/s, or 9.09-23.68 km/h |
| Wind speed range from current base/gust | 2.2-5.8 m/s, before max(0, ·) |
| Bias probability at 4.0 m/s base wind | 0.32, or 32% |
| Bias probability range at 2.2-5.8 m/s | 0.176-0.464, or 17.6%-46.4% |
| Strong downwind jump at 5.8 m/s | 12.5 + 5.8·0.68 = 16.444 m |
| Strong downwind equivalent hop speed | 8.65 m/s, or 31.16 km/h |
| Strong upwind minimum after clamp | 2.88 m |
| Max hotspot creation rate | 1 / 1.9 = 0.526 hotspots/s |

Candidate hotspots are rejected if they are outside bounds, in already burned cells, on invalid ground, or fail optional tree-clearance checks. In current code tree clearance is ignored for spread because FIRE_SPREAD_IGNORE_TREE_CLEARANCE is True.

# Detection, Mapping, And Suppression

## Hotspot Detection Probability

d_h = sqrt((x_hotspot - x_drone)² + (y_hotspot - y_drone)²)
h_rel = z_drone - z_hotspot_ground
range_factor = max(0, 1 - d_h/R_det)
altitude_factor = max(0.25, min(1, h_rel/(2.5h_min)))
P_det = sat(λ_det · range_factor · altitude_factor · Δt, 0, 0.95)

Detection is impossible if d_h > 15 m or h_rel < 4.5 m. Automation also has a forced-detection rule: if an automated survey drone comes within 1.75 m of an active hotspot, it is marked detected immediately.

For repeated frames with changing P_det, the cumulative probability of at least one detection over n frames is 1 - Π(1 - P_det,i), assuming independent random draws.

## Mapping Coverage

f_map,i = min(1, mapping_progress_i / 12 s)
coverage_score = (1/N) Σ_i f_map,i

Mapping progress increases only for unresolved detected hotspots when at least one survey drone is near the 10.5 m orbit radius, within ±3.0 m, and above the 4.5 m scan-altitude threshold. Each qualifying survey drone adds dt seconds of map progress per frame.

## Suppression Lifecycle

r_supp = 0
if detected and detection_age ≥ 8 s: r_supp += 0.55
if water drone engaged: r_supp += 0.9 · n_water_engaged
W_i ← W_i + r_supp Δt

ACTIVE → CONTAINED if W_i ≥ 30 s
CONTAINED → OUT if W_i ≥ 30 s + 54 s = 84 s
ACTIVE → BURNED if burn_age ≥ 180 s
OUT → ACTIVE with probability 0.0035 Δt per frame

The variable W_i is suppression work, not wall-clock time. With only ground firefighters engaged, containment takes about 30/0.55 = 54.5 s after the response delay. With one water drone plus ground crew, the rate is 1.45 s/s and containment takes about 20.7 s of engagement.

## Fire Visual Growth Scale

T_growth = max(1, min(180, 30 · 1.15)) = 34.5 s
t_active = burn_age + min(detection_age, 30)
p_growth = min(1, t_active/T_growth)
scale = 0.82 + (1.38 - 0.82)p_growth

if contained:
  p_contained = sat((detection_age - 30)/54, 0, 1)
  scale ← scale · (1 - 0.34p_contained)

This affects presentation scale, not the mathematical spread probability directly.

# Automation And Patrol Formulas

## Survey Sectors And Boustrophedon Waypoints

S = max(1, team_survey_drone_count)
sector_width = (SPAWN_X_MAX - SPAWN_X_MIN) / S
x_min,j = SPAWN_X_MIN + j · sector_width
x_max,j = x_min,j + sector_width, except final sector uses SPAWN_X_MAX

Inside each sector, the patrol route is a boustrophedon sweep: x positions advance by 11 m spacing, and the target y alternates between the sector's lower and upper interior margins.

## Survey Orbit

ω_orbit = v_orbit/R_orbit = 7.5/10.5 ≈ 0.714 rad/s
phase ← (phase + ω_orbit Δt) mod 2π
x_orbit = x_hotspot + R_orbit cos(phase)
y_orbit = y_hotspot + R_orbit sin(phase)

The drone then flies toward the moving orbit point with speed scaled by distance/R_orbit and clamped to the automation speed limit.

## Automation Target Velocity And Altitude

if d_target > R_arrival:
  speed_scale = min(1, d_target/R_det)
  v_des,h = unit(target - drone) · v_h,max · speed_scale
else:
  v_des,h = 0

z_des = max(22.0, z_target + 4.5, z_ground + 8.0, z_canopy + 6.0)
v_z,des = sat(0.9(z_des - z), -v_z,max, v_z,max)

Tree avoidance and inter-drone avoidance can add lateral velocity corrections before final speed clamping. Those corrections are engineering safety layers, not paper quadrotor equations.

# Performance Matrix

## Team Effectiveness

E_role(n) = sat(1 + 0.16 max(0, n - 1), 0.35, 1.45)
bonus = min(0.12, 0.018 max(0, N_total - 2))

Survey effectiveness multiplies detection and coverage terms. Water effectiveness multiplies suppression and fire-out terms. The small extra-drone bonus increases the score ceiling for larger teams.

## Detection Score M1

δ_i = detection_time_i - ignition_time_i, if detected
δ_i = current_time - ignition_time_i, if not yet detected
s_det,i = 1 - sat(δ_i / 8 s, 0, 1)
detection_score = (1/N) Σ_i s_det,i
M1 = sat(detection_score · E_survey, 0, 1)

The 8 s target comes from 120 real minutes divided by 15 real-minutes-per-sim-second. The prototype later overrides suppression pacing, but not this detection target.

## Coverage, Suppression, And Fire-Out Scores

M2 = sat(coverage_score · E_survey, 0, 1)
M3 = sat(suppression_progress_score · E_water, 0, 1)
M4 = sat(fire_out_control_score · E_water, 0, 1)

suppression progress per detected hotspot:
OUT: 1
BURNED: 0
CONTAINED: 0.5 + 0.5·sat((W_i - 30)/54, 0, 1)
ACTIVE: 0.5·sat(W_i/30, 0, 1), with ground-crew baseline max before the 0.5 factor

fire-out control per hotspot:
OUT: 1
CONTAINED: 0.65 + 0.35·sat((W_i - 30)/54, 0, 1)
ACTIVE and detected: sat(W_i/84, 0, 0.35)
undetected or burned: 0

## Composite Score

raw_score = sat((0.30M1 + 0.20M2 + 0.20M3 + 0.30M4) / 1.0, 0, 1)
performance_score = sat(raw_score + bonus, 0, 1)
reward eligible if performance_score ≥ 0.72

M1 and M4 have the largest weights in the current code. That makes fast discovery and actually putting fires out more important than partial mapping or partial suppression progress.

# Corrections Compared With The Older Notes

Current motor lag is τ_motor = 0.075 s. Any old mention of 0.03 s is no longer correct.

Current fire spread interval is 1.9 s, not 1.2 s.

Current spread events per tick is 1, not 2.

Current base spread distance range is 4.8-12.5 m, not 10-22 m.

Current wind directional-bias factor is 0.08 per m/s, not 0.11.

Current downwind distance gain is 0.68 and upwind penalty is 1.0.

The old main.py line references around 34xx-35xx are incomplete because the lead-drone update now lives around 8640-8960 and the reusable follower function around 3303-3595.

The simulator should be described as paper-inspired with direct matches in key equations, not as a full line-by-line implementation of the paper model.

# Concise Defense Statement

The current simulator is technically defensible as a low-to-mid-fidelity operational demo. It uses real Euler-angle attitude states, torque-to-rotor mixing, first-order actuator lag, thrust-projected motion, wind-biased discrete fire spread, probabilistic detection, explicit suppression work, and a transparent weighted performance score. Its main limitations are equally explicit: it does not implement the full Euler/body-rate transform, full DC motor transfer function, full A/B state-space matrices, continuous wildfire physics, sensor false positives/negatives beyond probability gating, or high-fidelity aerodynamics.